VISEON C11 TECHNICAL DATA

Dimensions and weights

Length	11,375 mm

Width 	2,550 mm

Height	3,560 mm

Standing headroom (centre aisle)	min. 1,970 mm

Boarding height (front/rear)	350/350 mm

Wheel base	5,425 mm

Floor height	1,310 mm

Overhang (front/rear)	2,560/3,300 mm

Min. theoretical turning circle	19,700 mm

Tare weight (equipment-dependent)	12,300 - 12,900 kg

Perm. gross weight	18,000 kg

Tech. permissible gross weight

Front axle / rear axle	7,100 / 13,000 kg

Total	19,700 kg

Luggage compartment rear entry version	9.3 m³

Luggage compartment middle entry version	8.7 m³

Fuel tank capacity	320 l

Optional additional tank	220 l

Passenger seating middle entry version

3-star seating up to 45 + 1 + 1

4-star seating up to 40 + 1 + 1

5-star seating up to 36 + 1 + 1

Passenger seating rear entry version

without kitchen   3-star seating up to 43 + 1 + 1

with kitchen        3-star seating up to 40 + 1 + 1

with kitchen        4-star seating up to 38 + 1 + 1

with kitchen        5-star seating up to 34 + 1 + 1

Engine

Water-cooled six cylinder in-line diesel engine (MAN D2066 LOH), installed vertically, with turbo charging and charge air cooling, electronically controlled direct fuel injection with Common Rail System, cooled exhaust gas recirculation, closed particle filter, EEV exhaust emission standard.

Bore/stroke 112/155 mm, displacement 10,518 cm³

Compression ratio 17:1

Engine power 294 kW (400 PS) at 1,900/min

Maximum torque 1,900 Nm at 1,000 – 1,400/min

Optional

Water-cooled six cylinder in-line diesel engine (MAN D2676 LOH), installed vertically, with turbo charging and charge air cooling, electronically-controlled direct fuel injection with Common Rail System, cooled exhaust gas recirculation, closed particle filter, EEV exhaust emission standard.

Bore/stroke 126/166 mm, displacement 12,419 cm³

Compression ratio 17:1

Engine power 324 kW (440 PS) at 1,900/min

Maximum torque 2,100 Nm at 1,000 – 1,400/min

Power train

Transmission (with D2066 LOH): synchronised 6-speed manual gearbox ZF 6 S 1901 with integrated retarder, manual operations via pneumatically-assisted cable-controlled gearshift.

Transmission ratios:	6.32 – 0.81.

Optional transmission: automated 12-speed gearbox

ZF 12 AS-Tronic 2301 BO with integrated retarder. Operated via ZF driving switch (joystick). Transmission ratios 12.33 - 0.78.

Chassis

Independent wheel suspension at the front ZF RL 75 E, trapezoid multi-link axle, two pneumatic spring bellows, two shock absorbers, stabilizer, technical permissible axle load 7.1 t. Rigid drive axle ZFA132 at rear, two trailing arms, overhead triangular trailing arms, four air spring bellows, four shock absorbers, optional stabilizer, technical permissible axle load 13 t. Tyres 295/80 R 22.5.

Axle drive ratio i = 3.54 as standard

Optional in connection with ZF AS-Tronic i = 3.23

Braking system

Electronically-controlled dual-circuit brake installation with all-round ventilated disc brakes, ZF Intarder permanent brake, operated via the steering lever connected upstream of the foot brake, retarder brake limiter, ABS and ASR, brake assistant, ESP Electronic Stability Program (WABCO-ESC).

Steering

ZF ball-and-nut-type power steering gear Type 8098 Servocom with variable transmission ratio (17.1 – 20.1), direct transmission ratio with approx. 2.4 turns of the steering wheel, hydraulic torque at 170 bar 6,478 - 7,621 Nm, steering column pneumatically adjustable in both height and tilt.

Electrics

Voltage 24V, 2 three-phase generators à 28,5 V / 120 A, two batteries 12 V / 225 Ah.

Front lighting

Halogen headlights for dipped and full beam as standard, optional bi-xenon lamps for dipped and full beam (including headlight cleaning system), daytime lamps, indicator and marker lights (on roof) in long-life LED technology, fog lamps with cornering lights.

Rear lighting

Tail lights, brake lights, indicators, reversing lights and rear fog lamps in LED technology, multi-compartment tail lamps (tail-indicator-brake lamps) at top also with LED technology.

Miscellaneous

LED licence plate lighting, side marker lights and 1 indicator light on the left and right side of the vehicle respectively in long-life LED technology, lighting for luggage compartments also with LED technology.

Heating/ventilation/air-conditioning

Fully-automatic heating and ventilation system with roof-mounted air-conditioning system. Draught-free room ventilation, air intake via slits in roof channel in the direction of the windows and centre aisle. Electrically-operated skylight. Ventilation via the luggage compartment. Webasto auxiliary heating, 35W; maintenance-free convector heating system in passenger area, additional heater at the front 15 kW, cooling capacity 6 kW.

Roof-fitted air-conditioning system:

Air-conditioning ***

Cooling capacity : 24 kW

Heating power : 38 kW

Ventilation performance : 4,300 m³/h

Optional

Air-conditioning **** (Hot zone)

Cooling capacity : 32 kW

Heating power : 38 kW

Ventilation performance : 6,450 m³/h